# Dag 1

MQ135 aan arduino gekoppeld

4 uitgangen sensor:		naar arduino:
 	- A0				A0
	- D0				pin 2 (of andere, specificeren in code)
	- GND		 		GND
	- VCC				5V

Via breadboard kunnen de 3 sensoren snel worden gewisseld

Voor de kalibratie van deze sensoren moeten er een paar dingen gebeuren, eerst moeten ze minstens 24u aan staan om voorverwarmd te worden. Hierna moet R0 worden berekend, dit kan via

#include "MQ135.h"
const int ANALOGPIN=0;
MQ135 gasSensor = MQ135(ANALOGPIN);
void setup(){
Serial.begin(9600);
}
void loop(){
float rzero = gasSensor.getRZero();
Serial.println(rzero);
delay(1000);
}

Hierna waarde aanpassen in MQ135.h file

#include "MQ135.h"
const int ANALOGPIN=0;
MQ135 gasSensor = MQ135(ANALOGPIN);
void setup(){
Serial.begin(9600); 
}
void loop(){
float ppm = gasSensor.getPPM();
Serial.println(ppm);
delay(1000);
}

Elke keer dat uw sensor in een vooraf gedefinieerde concentratie met dat gas in aanraking komt, wordt DO laag (0V), anders blijft het hoog (5V).

# Dag 2

De sensoren hebben elk 24u kalibratietijd gekrijgen

Op basis van Khadim, H. J., Obaed, F. K., & Ali, Z. T. A heb ik 3 sensoren gekoppeld

int MQ4;
int MQ135;
int MQ7;

void setup()
{
  Serial.begin(9600); // sets the serial port to 9600
  pinMode(13, OUTPUT);
  pinMode(2, INPUT);
}

void loop()
{
  MQ4 = analogRead(0); // read analog input pin 0
  MQ135 = analogRead(1); // read analog input pin 1
  MQ7 = analogRead(2); // read analog input pin 2

  Serial.print("MQ4 waarde = ");
  Serial.println(MQ4, DEC);

  Serial.print("MQ135 waarde = ");
  Serial.println(MQ135, DEC);

  Serial.print("MQ7 waarde = ");
  Serial.println(MQ7, DEC);

  delay(1000); // wait 100ms for next reading
}

# Dag 3

KALIBRATIE:

https://en.wikipedia.org/wiki/Log%E2%80%93log_plot

https://nl.wikipedia.org/wiki/Wet_van_Ohm

https://www.onetransistor.eu/2022/12/mq-sensors-compute-gas-ppm.html

https://www.sciencedirect.com/science/article/pii/S187705092030051X?ref=pdf_download&fr=RR-2&rr=87af1a6a68e52e08

https://www.researchgate.net/profile/Mihaela-Hnatiuc/publication/330597436_Acquisition_and_Calibration_Interface_for_Gas_Sensors/links/5eaaa60992851cb26766db30/Acquisition-and-Calibration-Interface-for-Gas-Sensors.pdf

https://www.jaycon.com/understanding-a-gas-sensor/

PDF: MQ135 studie met berekeningen

https://www.rapidtables.com/math/algebra/Logarithm.html

https://www.kalaharijournals.com/resources/FebV7_I2_372.pdf

Waarden met webplotdigitizer:

Voor te citeren  
  @software{Rohatgi2024,
        url = {https://automeris.io/WebPlotDigitizer.html},
        author = {Rohatgi,  Ankit},
        title = {WebPlotDigitizer},
        version = {4.7},
        year = {2024}
    }

MQ4:

https://www.sparkfun.com/datasheets/Sensors/Biometric/MQ-4.pdf

Rs/R0 bij lucht = 4,4

| CO | 230,6978134266655; 4,231157034510028 9369,990065629532; 3,5159614004010766 |
| --- | --- |
| Alcohol (C₂H₆O) | 218,33855652562346; 4,0097088626504025 9092,758408292557; 3,0830130048242466 |
| rook | 223,87211385683378; 3,8917297279993073 9138,387404950741; 2,577236924966879 |
| H2 | 222,75429519995564; 3,7101408910120584 9276,6528408599; 1,923298498734063 |
| LPG | 224,99554191643315; 2,47170199976446 9184,24537561687; 0,7574909209837059 |
| CH4 | 222,75429519995564; 1,6864674006602332 9184,24537561687; 0,4532006580364012 |

MQ135:

https://www.electronicoscaldas.com/datasheet/MQ-135_Hanwei.pdf

Rs/R0 bij lucht = 3,6

| CO | 10,722672220103231; 2,784348665061436 191,15483236201203; 1,3509935211980189 |
| --- | --- |
| NH4 | 10,616320618413; 2,4945081352303013 190,20449633640098; 0,7797579279269474 |
| CO2 | 10,722672220103231; 2,2478058335487074 188,31797472883628; 0,8166957611638451 |
| Alcohol (C₂H₆O) | 10,51102385295668; 1,8679135990207614 189,25888495494576; 0,7488103857589935 |
| Tolueen (C₆H₅CH₃) | 10,458767760290366; 1,525517118540575 188,31797472883628; 0,6555154234890879 |
| Aceton (C₃H₆O) | 10,722672220103231; 1,398713102647215 189,25888495494576; 0,5975605379290404 |

MQ7:

https://www.sparkfun.com/datasheets/Sensors/Biometric/MQ-7.pdf

Rs/R0 bij lucht = 25,8

| Alcohol (C₂H₆O) | 56,12645227692062; 15,884971372828545 3525,0867676009134; 12,233373604997416 |
| --- | --- |
| CH4 | 56,55841435770048; 13,704653643190992 3634,8655073431873; 9,20961927662913 |
| LPG | 53,193694579265916; 8,901129704146443 3498,164092480776; 5,10232581359145 |
| CO | 53,603085482533054; 1,5136471967103977 3552,216645818579; 0,09803218456661832 |
| H2 | 54,85025997414313; 1,1924737525841909 3525,0867676009134; 0,056836054376472406 |

# Dag 4

## Implementatie van heat/cool cycle bij MQ7

Volgens de datasheet werkt de MQ7 (CO) het best bij een heat/cool cycle, hier wordt de sensor doorheen 2 cyclussen gecycled, tijdens de lage temperatuurfase (1,4V voor 90 seconden) wordt CO geabsorbeerd op de plaat, wat betekenisvolle gegevens oplevert.  Tijdens de hoge temperatuurfase (5V voor 60 seconden) verdampen geabsorbeerde CO en andere verbindingen van de sensorplaat, waardoor deze wordt gereinigd voor de volgende meting.

I've looked into this a bit, and here are my comments:

- The MQ-7 module is so poorly documented that it is not a great choice to work with

- There are other (qualitative) sensors (such as the mics5524) that seem simpler to work with, and don't appear to have any clear disadvantage vs. the MQ-7.

- If I were to take the time to calibrate the MQ-7 fully, it would only really be of use to me. Unless I mapped out the entire RH, temperature, CO concentration, and interfering gases profile, it is unlikely my results would be directly applicable to anyone else. Additionally, even if I did so, anyone looking to make quantitative measurements would (hopefully) repeat some of my results under their exact conditions with their own sensor to ensure they got the same data. By the time this was complete, their own data would be more useful for them than what I could provide.

- In case it is of help to anyone, I am posting the circuit that was suggested by Leo. I have built this circuit, and it worked exactly as expected (thanks Leo). While I did physically build the circuit, for this group, I am also posting the results of the circuit simulation in case anyone finds it useful.

https://www.instructables.com/Arduino-CO-Monitor-Using-MQ-7-Sensor/

pdf: CalibrationandimplementationofMQ7_.pdf

## Implementatie van ESP8266 wifi-module

https://www.youtube.com/watch?v=1LJgZkrKMeo&t=1s

https://www.instructables.com/IoT-Made-Easy-With-UNO-ESP-01-ThingSpeak-and-MIT-A/

https://arduino.stackexchange.com/questions/48402/unable-to-upload-the-data-on-thingspeak-using-uno-and-esp8266

esp updaten: https://wiki.aprbrother.com/en/Firmware_For_ESP8266.html

COMMANDOS:

AT

AT+CWMODE=1

AT+CWJAP="Wifinetwerk van Ludo","Kamiel2003"

AT+CIPMUX=0

AT+CIPSTART="TCP","184.106.153.149",80

AT+CIPSEND=44

GET /update?key=MRUMJIBRR5SBF98K&field1=20

https://www.espressif.com/sites/default/files/documentation/4b-esp8266_at_command_examples_en.pdf

gebruiken om CIPMUX in te stellen (pagina 7)

## Implementatie van databank

MySQL + php file (via XAMPP)

AT+CIFSR (ip krijgen van ESP01)

Php files in C:\xampp\htdocs

Request maken via
http://127.0.0.1/insertMQ4.php?MQ4_CO_ppm=1&MQ4_alcohol_ppm=2&MQ4_rook_ppm=3&MQ4_H2_ppm=4&MQ4_LPG_ppm=5&MQ4_CH4_ppm=6

https://youtu.be/-FLer3NmePs?si=m7rpbxuoI3a8inSn

https://it-g-house.tistory.com/entry/%EC%95%84%EB%91%90%EC%9D%B4%EB%85%B8-%ED%99%88IoT-%EC%84%9C%EB%B2%84-%ED%99%9C%EC%9A%A9-ESP8266ESP-01-%EC%9D%B4%EC%9A%A9%ED%95%98%EC%97%AC-DB%EC%84%9C%EB%B2%84MySQL%EC%97%90-%EC%98%A8%EC%8A%B5%EB%8F%84-%EB%8D%B0%EC%9D%B4%ED%84%B0-%EC%A0%80%EC%9E%A5%ED%95%98%EA%B8%B0

AT+CIPSTART="TCP","192.168.1.44",80
CONNECT
OK

AT+CIPSEND=236
OK
> POST /insertMQ4DB.php HTTP/1.1
Host: 192.168.1.44
Connection: keep-alive
Content-Type: application/x-www-form-urlencoded
Content-Length: 86

MQ4_CO_ppm=1&MQ4_alcohol_ppm=2&MQ4_rook_ppm=3&MQ4_H2_ppm=4&MQ4_LPG_ppm=5&MQ4_CH4_ppm=6

SEND OK
+IPD,285:HTTP/1.1 200 OK
Date: Tue, 16 Apr 2024 14:54:11 GMT
Server: Apache/2.4.58 (Win64) OpenSSL/3.1.3 PHP/8.2.12
X-Powered-By: PHP/8.2.12
Content-Length: 31
Keep-Alive: timeout=5, max=100
Connection: Keep-Alive
Content-Type: text/html; charset=UTF-8

New record created successfully
CLOSED

!!!!!!!!!!! joepieeeeee !!!!!!!!!!!!!! eindelijk !!!!!!!!!!!!!!!

## Implementatie van de DHT22

https://www.ardumotive.com/how-to-use-dht-22-sensor-en.html

https://www.researchgate.net/publication/328875972_Influence_of_Temperature_and_Humidity_on_the_Output_Resistance_Ratio_of_the_MQ-135_Sensor

toont kalibratiefactor aan

Vanaf 20°C ongeveer lineair

MQ135:

via webplotdigitizer data uit grafiek halen en via curve fitting a, b en c vinden

Vanaf 20°C niet meer correct  geen 2e graad meer maar 1e graad

aparte curve fit

MQ4 under 20 a: 0.00021598016836642446 b: -0.011780605617435404 c: 1.1489572784939854
MQ4 over 20 a: -0.00295586181504405 b: 1.0533607677395043

MQ135 under 20 a: 0.00046307428153304244 b: -0.02906738438091286 c: 1.3784941304658396
MQ135 over 20 a: -0.002329525704310693 b: 1.026785488456616

MQ7 under 20 a: 0.0004604255602040132 b: -0.019452899554979888 c: 1.2135189729083746
MQ7 over 20 a: -0.004424792057792776 b: 1.0851404282394788

https://youtu.be/Tz_aXlJEHcw?si=CguS7ytBNTOgnUbJ
 multiple regression in python